| Step | Script / Command / Software | Input data | Input parameters | Output |
| --- | --- | --- | --- | --- |
| 1 | Split Multipoints (NIS Elements) | Multipoint time series (ND2 file) | NA | XY points (CZT ND2) |
| 2 | split_time_batch.py (Fiji) | Batch of XY points from one movie (from step 1) | # timepoints | Individual timepoints (CZ tiff) |
| 3 | max_project_batch.ijm (Fiji) | Batch of timepoint Z stacks from step 2 (can be from multiple runs) | NA | Max projections (multichannel tiff) |
| 4 | Rbody_localization.cpproj or Rbody_localization_lyso.cpproj (CellProfiler) | Max projections from step 3 |  | Counts of Rbodies ≥ with 50% of area overlapping with vesicles in each max projection; outlines of Rbodies, cells, vesicles, and colocalized objects |

Plot: Fraction of cellular Rbodies in vesicles vs timepoint

If significant coloc detected, investigate further (single slice or 3d analysis)

Possible improvement: Use max proj for cell detection, single slice for Rbody and vesicle detection (start with completely split dataset)