Workflow with single slices

| Step | Script / Command / Software | Input data | Input parameters | Output |
| --- | --- | --- | --- | --- |
| 1 | Split Multipoints (NIS Elements) | Multipoint time series (ND2 file) | NA | XY points (CZT ND2) |
| 2 | File > Save As … > Image Sequence | Single XY ND2 files from one movie (from step 1) | # digits for numbering (3); dimension order (CZT?) | Individual planes (C, Z, T) in format XXX_t000_z000_c000.tif |
| 3 | split_time_batch.py (Fiji) | Batch of XY points from one movie (from step 1) | # timepoints | Individual timepoints (CZ tiff) |
| 4 | max_project_batch.ijm (Fiji) | Batch of timepoint Z stacks from step 3 (can be from multiple runs) | NA | Max projections (multichannel tiff) |
| 4 | For endosomes: Rbody_localization_slice.cppipe For lysosomes: Rbody_localization_lyso_slice.cpproj (CellProfiler) | Max projections from step 4; single plane images from step 2 (can be from multiple runs) | Set slice number in Names And Types step; set overlap % in Filter Objects step | Counts and measurements of Rbodies overlapping with vesicles in each slice; outlines of Rbodies, cells (based on max proj), vesicles, and colocalized objects (based on single slice) |

Workflow with max projections

| Step | Script / Command / Software | Input data | Input parameters | Output |
| --- | --- | --- | --- | --- |
| 1 | Split Multipoints (NIS Elements) | Multipoint time series (ND2 file) | NA | XY points (CZT ND2) |
| 2 | split_time_batch.py (Fiji) | Batch of XY points from one movie (from step 1) | # timepoints | Individual timepoints (CZ tiff) |
| 3 | max_project_batch.ijm (Fiji) | Batch of timepoint Z stacks from step 2 (can be from multiple runs) | NA | Max projections (multichannel tiff) |
| 4 | Rbody_localization.cpproj or Rbody_localization_lyso.cpproj (CellProfiler) | Max projections from step 3 |  | Counts and measurements of Rbodies overlapping with vesicles in each slice; outlines of Rbodies, cells, vesicles, and colocalized objects (based on max proj) |